Samenvatting Harris 18 (en 20.6)

18.1-3, 18.5, 18.6, 20.6

18.1)

- νλ = c

- E = hν

- 1/λ = wavenumber ↔ E

- De verschillende golflengtes hangen samen met fenomenen die de energie veroorzaken (zie figuur 18.2 voor een overzicht).

18.2)

- Excited state/ground state.

- When light absorbed, P, energy/second/unit area, irradiance of beam decreased.

- Monochromatic = one color.

- Transmittance T:

  - T = P/P0.

- Absorbance A:

  - A = log (P0/P) = -log T.

- Absorbance is directly proportional concentration, Beer(-Lambert)’s law:

  - A = εbc. c usually moles/liter, pathlength b centimetres and ε, quantity, is called molar absorbitivity (aka extinction coefficient), tells how much light absorbed certain wavelength, given in /M/cm. Absorbance dimensionless. Also Aλ = ελbc.

- Absorption spectrum: showing how A varies with λ.

- Chromophore: part of molecule responsible light absorbance.

- Color absorbed → substance seen as complement coloured.

- Beer’s law applies to monochromatic radiation and <0.01M. Not when:

  - When molecules get close properties change.

  - At high concentration dilute becomes solvent → properties change different solvents.

  - Nonabsorbing solutes can also interact absorbing species.

  - Absorbing molecule participates in concentration dependent chemical equilibrium.

18.3)

- Spectrophotometer: light source - monochromator – sample – light detector.

- Cuvet, base line/blank.

- In spectrometric analysis, wavelength max absorption used because:

  - Sensitivity greatest at max absorbance (max response)

  - Curve is relatively flat at max → little variation.

- Spectrometers most accurate at intermediate level absorbance (A ≈ 0.4-0.9):

  - Too little light (high absorbance): intensity hard to measure.

  - Too much light (low absorbance): hard to distinguish difference sample/ref.

- Detection limit by reproducibility measurement.

  - Avoid dust (→ “absorbance”).

  - Filter solution.

  - Cuvet clean.

18.5)

- Molecular orbitals, electronic transition.

- Two possible spin states:

  - opposed: singlet state (lowest is called S1)

  - parallel: triplet state (lowest is called T1) (named because of magnetic field)

- Vibrational (>ΔE) and rotational (< ΔE) energy levels.

- What happens to absorbed energy:

  - Vibrational relaxation (to lowest vibrational level) by radiationless transition (trough collisions, e.g. with solvent): heat spread to the medium.

  - Internal conversion: enter highly vibrational level of Sn-k. (From here on relaxation).

  - Intersystem crossing: e.g. from S1 to T1.

  - Relax by emitting a photon

    - S1 → So: fluorescence

    - T1 → So: phosphorescence.

    - Lifetime of fluorescence shorter then phosphorescence. (lifetime: time needed population decay to 1/e)

Relative rates depend on molecule, solvent and conditions (T, P). Fluor- and phosphorescence relatively rare.

18.6) Luminescence

- Luminescence: emission of light from an exited state of a molecule (e.g. fluorescence & phosphorescence).

- Luminescence more sensitive absorption (absorption vs. background harder to detect luminescence vs. background).

- Emission spectrum: exciting λ constant, intensity emitted vs emitted wavelength.

- Excitation spectrum: measuring emitted λ constant, intensity emitted vs exciting λ.

- Detector response varies λ.

- At low analyte [] emission proportional (formulas: p. 396).

20.6) Choosing monochromator bandwith

- Monochromator bandwidth (#λ; ≠ wavelength) 1/5 peak gives peak shape, largest possible preferred.