Samenvatting Harris 23

23.1) (solvent extraction)

23.2)

- Mobile phase (moving) and stationary phase (stays). Elution (passing liquid, through column): eluent (in) → eluate (out). Packed (filled stationary phase) vs tubular (hollow).

- – 508. Chromatography: (1) adsorption ~ (solid stationary, liquid/gas mobile; adsorption↑ → traveltime↑), (2) partition ~ (liquid stationary, gas mobile), (3) ion exchange ~ (anions covalently attached resin solid stationary state, attract opposite charge, fluid mobile), (4) molecular exclusion ~ (= gel filtration / gel permeation; separates size porous gel; small take longer), (5) affinity ~ (covalently attached second molecule to stationary phase with affinity, e.g. antibody, dislodging by pH or ionic strength).

23.3)

- Volume flow rate (mL/min), linear flow rate (cm/min). Chromatogram (graph detector response).

- – 510. Retention time (tr), retention volume (Vr; volume mobile phase required elute solute), minimum possible time (tm), adjusted retention time (t’r = tr – tm), relative retention (α = t’r2/t’r1 = k’2/k’1 = K2/K1 > 1), each peak capacity factor (k’ = t’r/tm = timestationary/timemobile = molesstationary/molessolute).

- Vr = tr∙uv (retention volume = retention time∙volume flow rate). Analytical vs preparative. Scaling ↔ cross-section area.

23.4)

- – 512. Degree separation influenced Δelution time & broadness peak. Residence time↑ → peaks broader. Resolution: Δtr/wav = ΔVr/wav = 0.589Δtr/w1/2av (Δtr or ΔVr = separation). W = 4σ.

- Band spreading by diffusion, σ = √(2Dt). J = -D(dc/dx) (Flux across plane ↔ concentration & D; diffusion coefficient).

- Plate height: H = σ2/x, (variance/distance travelled; historical, measure column efficiency). Plate height↓ → bandwidth↓. N (number of plates on column) = 16tr2/σ2 = 5.55tr2/w1/22 = L/H. (..)

- Resolution = √(N)/4∙(γ-1); γ (= V1/V2 > 1; velocities; separation factor).

- (table 23-2: summary equations).

- (addition, ..)

- – 519. Van Deemter equation:

H ≈ A + B/ux + Cux ux: linear flowrate; A (multiple paths), B (longtidunal diffusion), C (equilibration time): constants.

Broadening due: (1) multiple paths (multiple effects simplified; “different flow paths”), (2) Longtidunal diffusion (dependent flowrate), (3) equilibration time (stationary phase “gets left behind”).

| Open tubular column | vs | Packed column |
| --- | --- | --- |
| Higher resolution / Shorter analysis time Higher sensitivity Lower capacity |  | Lower resolution Preparative |

- (Asymmetric band shapes).