Samenvatting Harris 24

24.1-24.4 after powerpoint

24.1)

- Gass chromatography (GC): carrier gas (gaseous mobile phase). Gas-liquid partition ~ (GLC) (involatile) vs gas solid adsorption ~ (GSC).

- N ↔ length: resolution ↔ √L.

- “Like dissolves like”: nonpolar analyte → nonpolar column, etc.

- Common stationary phases capillary gas chromatography:

| structure | polarity | T range |
| --- | --- | --- |
| (Diphenyl)x(dimethyl)1-xpolysiloxane | x = 0: nonpolar x = 0.65: intermediate | 60 - 320˚C 50 - 370˚C |
| (Cyanopropylphenyl)0.14(dimethyl)0.86polysiloxane | Intermediate | 20 - 280˚C |
| Carbowax(poly(ethyleneglycol)) | Strongly polar | 40 - 250˚C |
| (Biscyanopropyl)0.9(cyanoropylphenyl)0.1polysoxane | Strongly polar | 0 - 275˚C |

- Particle size packed columns↓ → A↓ → R↑, also timeequilibration↑ → efficiency↑ but also less space → pressure required↑.

- (Retention index).

- Temperature effect volatility → temperature programming (T↑ x˚C/min).

- (..)

- Guard column (accumulate non-volatile substances) & retention gap (evaporate liquid).

24.2) (Sample injection)

24.3) Detectors

- Area Gaussian peak = 1.064∙height∙w1/2. Thermal conductivity detector (often with “analyte cooled” T-sensitive resistor): respond all, not sensitive.

- Flame ionization detector (eluate burned in H2 & air, C-atoms → radicals → CHO+ ions & e- → current): more sensitive, responds most hydrocarbons. Electron capture detector (high E e- from 63Ni→ analytes capture e-): sensitive halogen-containing molecules, less hydrocarbons etc.

- (Other specific detectors).

- (Gas chromatography mass spectrometry, element specific plasma detectors).

24.4) (Sample preparation).