Samenvatting Harris 25

25.1-25.2 after powerpointpresentation.

25.1)

- High-performance Liquid chromatography (HPLC; high pressure): ↓D → Packed columns.

- Smaller particles → number & length paths↓ → better resolution.

- T↑ → tr↓ → resolution↑. Microporous particles of silica generally used stationary phase.

- 8μmol silanol groups (Si-OH <pH3).

- 4μmol R groups (see right).

- Superficially porous particles (porous outer layer, solid core):  faster, used macromolecules.

- Adsorption chromatography (solute competes solvent), eluent strength (ε˚; measure solvent adsorption energy; ↑→rapid elution). Normal-phase chromatography (polar stationary, less polar solvent) vs reversed-phase chromatography (stationary weak/ nonpolar, solvent more polar). Polarity solvent → polarity stationary → ε˚↑.

- Isocratic (constant solvent) vs gradient (gradually changing solvent) elution. Latter ε˚↕.

- – 570. (..)

25.2) (Injection &) detection

  - (..)

  - Often UV detector.

  - Fluorescence detectors (excite with laser). Derivatization (attaching fluorescent or electroactive groups). Refractive index detector: responds all, poor detection limit.

  - Evaporative light-scattering detector (heated nebuliser evaporates solvent, analyte remains; responds analyte less volatile mobile phase).

  - Electrochemical detector (responds oxidizable or reducable analytes: potential → current).