Samenvatting Harris 26

After powerpointpresentation.

26.1) Ion-exchange chromatography

- Ion-exchange chromatography (retention attraction solute ions ↔ charged sites bound stationary phase). Anion (negative; solute) vs cation (positive) exchangers (opposite charge).

- – 594. (..)

26.2) (Ion chromatography)

26.3) Molecular exclusion chromatography

- Molecular exclusion chromatography (= size exclusion/gel filtration/gel permeation chromatography; pores stationary phase → smaller molecules take longer).

- – 601. (..)

26.4) Affinity chromatography

- Affinity chromatography (affinity → single compound binds column → eluted ↕conditions).

26.5) Capillary electrophoresis

- – 604. Capillary electrophoresis (electric field → migration; solutes different mobilities): only longtidunal diffusion v. Deemter (no multiple paths or equilibrium); often used complete cells (inserted or burst).

- Molecule moved forward qE (charge∙electric field), retarded frictional force fuep (friction coëfficient∙velocity). Electrophoretic mobility (μep) proportion speed vs strength field: uep = (q/f)∙E ≡ μepE (→ μep = uep/E).

- Electroosmosis (flow driven by excess cations, higher in concentration near negatively charged capillary wall >2pH).

- Electroosmotic mobility (μeo; ueo = μeoE): proportional surface charge density silica, inversely proportional √ionic strength. Flow ions → warming → lower viscosity (sometimes cooling). Apparent mobility ions (μapp) = μep + μeo and μapp = unet/E = (Ld/t)/(V/Lt) (Ld = length column injection-detector; t = migration time; V = voltage applied; Lt = complete length column). Determining electroosmotic flow → UV absorbing neutral solute. Speed analyte detector↕ → peak area/migration time.

- μeo = uneutral/E = (Ld/tneutral)/(V/Lt). Protein charge ladder (~same size different charge proteins).

- (..)

26.6) Conducting capillary electrophoresis.

- Hydrodynamic (ΔP between two ends capillary) vs electrokinetic injection (electric field drive sample → capillary).

- – 613. (conductivity effects: stacking & skewed bands)

- Ultraviolet detection, light pad short (bubble; angle bend: band broadening). Fluorescence detection, amperometric detection (oxidizable/reducable analytes), conductivity detection (small analyte ions), electrospray mass spectrometry (low detection limit, qualitative information). Indirect detection (substance with steady background signal, blocked analyte).

- – 617. Micellar electrokinetic chromatography (molecules equilibrate between micelles & free solution → ↕tr), micelles pseudostationary. (..)

- Capillary electrochromatography (capillary filled stationary phase). Capillarygel electrophoresis (cross linked polymers), separation by sieving. Polyacrylamide (DNA).

- Sodium dodecyl sulphate (reducing SS-bridges with 2-mercapthoethanol, dodecyl sulphate → hydrophobic regions → negative charge ↔ length protein).

- Iso-electric focussing (pH gradient & electric field applied → molecules → pI (pH where charge = 0) icw electrophoresis: 2D gel electrophoresis.